# CAN Display using Arduino nano &mcp2515

# Aim:

The objective is to interface a microcontroller (Arduino Nano) with a CAN controller (MCP2515) to receive battery state-of-charge data and compute the remaining range (DTE) of an electric vehicle. The calculated DTE is then displayed in real-time on a 4-digit 7-segment display (Catalex/TM1637), allowing users to monitor how far the vehicle can travel before the battery depletes.

# Principle:

This project is based on the principles of Controller Area Network (CAN) communication, battery energy estimation, and digital display control.

# Components:

Arduino Nano

4-digit 7-segment display (Catalex/TM1637Display)

Mcp2515 CAN Module

Battery

Arduino IDE

# Libraries to install:

TM1637TinyDisplay

Mcp_can

# Circuit Diagram:

# Hardware Connections:

| S.NO | MODULE USED | ARDUINO NANO | MODULE PINS |
| --- | --- | --- | --- |
| 1 | MCP2515 CAN Module | 5V | VCC |
| 2 |  | GND | GND |
| 3 |  | D10 | CS |
| 4 |  | D11 | MOSI |
| 5 |  | D12 | MISO |
| 6 |  | D13 | SCL |
| 7 |  | D2 | INT |

| S.NO | MODULE USED | ARDUINO NANO | MODULE PINS |
| --- | --- | --- | --- |
| 1 | Catalex/TM1637 | 5V | VCC |
| 2 |  | GND | GND |
| 3 |  | D3 | DIO |
| 4 |  | D2 | CLK |

# CODE:

#include <TM1637TinyDisplay.h>

#include <SPI.h>

#include <Wire.h>

#include <mcp_can.h>

#include <TM1637TinyDisplay.h>

const int SPI_CS_PIN = 10;

MCP_CAN CAN(SPI_CS_PIN);

#define CLK 2

#define DIO 3

TM1637TinyDisplay display(CLK, DIO);

unsigned long lastCanMsgTime = 0;

const unsigned long timeout = 2000; // 2 seconds timeout

void setup() {

Serial.begin(115200);

if (CAN.begin(MCP_ANY, CAN_500KBPS, MCP_8MHZ) == CAN_OK) {

Serial.println("CAN Bus Initialized");

} else {

Serial.println("CAN Bus Initialization Failed");

while (1);

}

CAN.setMode(MCP_NORMAL);

display.setBrightness(7);

display.showNumber(0);  // Show 00 at start

}

void loop() {

// Check for CAN message

if (CAN_MSGAVAIL == CAN.checkReceive()) {

unsigned long id;

byte len;

byte data[8];

CAN.readMsgBuf(&id, &len, data);

int idShort = id & 0xFFF;

if (idShort != 0x008) return;

// Update last received time

lastCanMsgTime = millis();

// Parse DTE

unsigned long remainingAhRaw =

((unsigned long)data[1] << 24) |

((unsigned long)data[2] << 16) |

((unsigned long)data[3] << 8)  |

((unsigned long)data[4]);

float remainingAh = remainingAhRaw * 0.001;

float remainingPower = (remainingAh * 51.2) - 230.4;

float DTE = remainingPower / 18;

Serial.print("DTE: ");

Serial.println(DTE);

// Display DTE (integer part only)

int dteInt = (int)DTE;

display.showNumber(dteInt);

}

// Timeout check: if no CAN message in `timeout` period

if (millis() - lastCanMsgTime > timeout) {

display.showNumber(0); // Show 00 when battery/CAN is inactive

}

}

# Formulas:

RemainingAh=(data[1]×2^24+data[2]×2^16+data[3]×2^8+data[4])×0.001.

RemainingPower (Wh)=(RemainingAh×51.2)−230.4.

DTE (km)=RemainingPower (Wh)/18​ .

🔧 Benefits

- Reduces need for a full CAN logger setup

- Compact and low-cost display module

- Easy to integrate into existing CAN systems

- Reusable and extensible for other CAN messages (e.g., motor control, temperatures)

⚠️ Challenges Faced in the Project

During the development of the CAN-based DTE (Distance To Empty) display system using Arduino Nano, MCP2515, and a TM1637 display, several challenges were encountered:

🔌 1. CAN Communication Initialization

- Issue: Difficulty in establishing stable communication with the MCP2515 module.

- Reason: Incorrect SPI settings or wrong crystal frequency (8 MHz vs. 16 MHz).

- Solution: Verified MCP2515 frequency and selected correct parameters: MCP_8MHZ in code.

📟 2. Display Not Updating

- Issue: TM1637 display was stuck showing 88:88 or garbage values.

- Reason: Wrong library used (TM1637TinyDisplay has limitations vs. TM1637Display).

- Solution: Matched display pins in code and used compatible library functions for numeric display.

📁 3. Library Compatibility Issues

- Issue: Errors like showNumberDecEx not recognized.

- Reason: That function exists in TM1637Display.h, not TM1637TinyDisplay.h.

- Solution: Adapted code to display integer-only DTE values or scaled decimal manually.

⚙️ 4. No CAN Data Received

- Issue: Display always showed 00 even though code was correct.

- Reason: Incorrect message ID filter or no actual CAN traffic.

- Solution: Ensured matching message ID (0x008), verified with serial monitor, and confirmed CAN sender functionality.

🚀 How This Project Can Be Improved in the Future:

To enhance the accuracy, functionality, and real-world usability of the CAN-based Distance To Empty (DTE) display project, here are key improvements and upgrades you can explore:

1. 🔋 Battery Data Accuracy Enhancement

- Current Method: DTE is calculated based on basic Ah and voltage.

- Improvement:

  - Add real-time current sensing (with INA219 or ACS712).

  - Factor in temperature and battery health from CAN data or sensors.

2. 📲 Mobile App Integration

- Improvement: Use Bluetooth (HC-05 or ESP32) to send DTE data to a smartphone.

- Display live DTE, battery status, and CAN errors in a user-friendly Android/iOS app.

3. 🧠 Intelligent DTE Estimation

- Improvement: Use machine learning models or adaptive algorithms that:

  - Learn from user driving patterns.

  - Adjust DTE in real-time based on speed, terrain (with GPS), and load.

4. 📦 Compact Custom PCB

- Improvement: Instead of using modules and jumper wires:

  - Design a custom PCB combining Arduino, MCP2515, and TM1637.

  - Increases reliability, reduces size, and is production-ready.

5. 🧾 Datalogging with SD Card

- Improvement: Add an SD card module to log:

  - SOC, DTE, current, voltage, temperature, errors.

  - Helpful for diagnostics, warranty tracking, and performance analysis.

6. 🗺️ GPS & Terrain-Based Adjustments

- Improvement: Integrate a GPS module (like NEO-6M) to:

  - Track real-time location.

  - Adjust DTE based on distance to next charging point or uphill/downhill.

7. ⚠️ Critical Alerts

- Improvement: Add LED indicators or a buzzer:

  - If DTE < 10 km.

  - For battery errors or temperature warnings.